1.Understanding the Sanitation Hygiene State (SHS)

The Sanitation Hygiene State (SHS) is a computed cleanliness score for a facility (like a restroom),

based on real-time sensor data. Instead of a person sniffing and deciding if the room is “clean” or

“dirty,” a processor uses sensor readings to calculate a numeric hygiene value. This gives an objective,

machine-measurable indicator of how clean the air is.

1.1 Sensing Odor-Causing Gases

The SHS system uses multiple gas sensors to capture smells or chemicals in the air that indicate

dirtiness. In particular, it measures:

● Ammonia (NH₃) – often present in urine odors.

● Hydrogen sulfide (H₂S) – smells like rotten eggs.

● Volatile organic compounds (VOC) – a catch-all for other smelly chemicals (denoted as V).

These three readings form a multi-gas concentration vector G(t) = [NH₃(t), H₂S(t), V(t)]. Each sensor

outputs an analog signal, which is converted into parts per million (ppm) for each gas.

1.2 Normalizing the Gas Readings

Raw sensor readings can vary widely, so the system normalizes them against known clean-air levels

and bad-odor thresholds. For each gas:

● Baseline (G_baseline): the normal concentration in clean conditions.

● Threshold (G_threshold): the concentration above which we consider the odor bad.

The reading G(t) is scaled using:

Normalized value = (G(t) – G_baseline) / (G_threshold – G_baseline).

This means when the reading equals the clean baseline, the normalized value is 0; when it equals the

threshold, the value is 1. In effect, each gas reading becomes a dimensionless deviation from clean air,

on a consistent scale.

1.3 Persistence: Ignoring Short-Term Spikes

A quick, temporary whiff of odor (like someone using the restroom briefly) shouldn’t immediately flag

the place as dirty. To avoid false alarms, the system uses a temporal persistence factor (P). This checks

if bad odors persist over a short time window (e.g. 60 seconds).

In practice:

● The system looks back over the last Δ seconds (say, 60 s).

● It counts how much of that time the combined odor level was above a “degradation threshold”

(T_d).

● If the odor level is high only for a few seconds, the persistence factor stays low.

● If the odor level stays high continuously (or often) over the window, P increases.

This makes SHS reflect sustained odors rather than fleeting spikes. In simple terms: “If it only smelled

bad for a moment, we don’t panic; if it stays bad for a minute, it matters.”

1.4 Adjusting for Environment

Humidity and temperature can affect gas sensor readings, so the SHS calculation includes an

Environmental Compensation Factor (ECF). The ECF is a linear combination of the current humidity

and temperature:

ECF(t) = k₁ * Humidity(t) + k₂ * Temperature(t).

Here, k₁ and k₂ are calibration constants determined in advance. This term essentially corrects the gas

readings so that the hygiene score isn’t thrown off by, say, a hot muggy day or a dry cold day.

1.5 Combining Factors into the SHS Score

After gathering and adjusting all pieces, the system computes the final hygiene score as a weighted

sum:

SHS(t) = w₁ * (Normalized NH₃) + w₂ * (Normalized H₂S) + w₃ * P(t) + w₄ * ECF(t).

● w₁, w₂: weights for the importance of ammonia and hydrogen sulfide levels.

● w₃: weight for the persistence factor (how much sustained odor influences the score).

● w₄: weight for the environmental compensation (how much we correct for

humidity/temperature).

All these w values are pre-set in the system (memory). The result SHS(t) is a continuous numerical

value. A higher SHS means worse air quality.1.6 Interpreting the SHS Value

The computed SHS number is then mapped to simple categories so the system can act on it

automatically. For example:

● SHS < 0.3: “Clean State.” The air is considered essentially fresh.

● 0.3 ≤ SHS < 0.6: “Moderate Degradation.” Some odor is present, but not severe.

● SHS ≥ 0.6: “Sustained Degradation.” The space is noticeably smelly and may need cleaning

soon.

These ranges and labels allow the system to trigger actions (like alerts or fan activation) in a clear way.

1.7 Why This Matters

By turning hygiene into a calculated number, the system achieves several benefits:

● Objective Measurement: Machines now “measure” cleanliness, replacing subjective human

sniff tests.

● Consistent Monitoring: Odors are tracked over time in a precise way, making it possible to

detect trends or chronic issues.

● Automated Responses: Cleaning staff or ventilation systems can be activated based on clear

metrics instead of guesswork.

● Data Tracking: The hygiene score can be logged over time, providing a “sanitation history” of

the facility.

In short, the SHS framework transforms “clean or dirty” from a vague judgment into a technical

parameter derived from sensors. This makes sanitation monitoring more scientific, reliable, and

efficient.

2.SENSOR FUSION LOGIC FOR SANITATION ASSESSMENT

Multi-Sensor Fusion Architecture

The present invention utilizes a processor-executed multi-sensor fusion architecture to determine

sanitation degradation within a monitored environment. Unlike conventional systems that rely on

independent threshold triggering from a single gas sensor, the disclosed system simultaneously

processes multiple bio-gas sensor inputs to generate a composite sanitation assessment.

All fusion logic is executed locally at an edge processor. Only validated sanitation events or computed

hygiene parameters are transmitted to a cloud-based monitoring platform. This architecture reduces

bandwidth usage, improves response time, and ensures that only verified sanitation states are

communicated externally.

2.1 Sensor Data Normalization

Each gas sensor generates an analog voltage proportional to the detected gas concentration. The

processor converts these raw outputs into calibrated parts-per-million (ppm) values using stored

calibration parameters.

Because different gases operate across different concentration ranges, direct aggregation would create

scale imbalance. To ensure comparability, each gas value is normalized according to the following

relation:

𝐺

𝑛𝑜𝑟𝑚

(𝑡) =

𝐺(𝑡)−𝐺

𝑏𝑎𝑠𝑒𝑙𝑖𝑛𝑒

𝐺

𝑡ℎ𝑟𝑒𝑠ℎ𝑜𝑙𝑑

−𝐺

𝑏𝑎𝑠𝑒𝑙𝑖𝑛𝑒

Where:

● 𝐺(𝑡)= real-time gas concentration

●

= reference concentration under clean conditions

𝐺

𝑏𝑎𝑠𝑒𝑙𝑖𝑛𝑒

●

= predefined sanitation degradation threshold

𝐺

𝑡ℎ𝑟𝑒𝑠ℎ𝑜𝑙𝑑

This normalization converts heterogeneous gas measurements into dimensionless values, allowing

accurate mathematical fusion across sensor types.

2.2 Weighted Aggregation of Gas Signals

Following normalization, each gas component is assigned a weighting coefficient reflecting its relative

sanitation significance. A composite fusion score is computed as:

𝑆

𝑓𝑢𝑠𝑖𝑜𝑛

(𝑡) = 𝑤

1

× 𝑁𝐻

3

𝑛𝑜𝑟𝑚 (𝑡) + 𝑤

2

× 𝐻

2

𝑆 𝑛𝑜𝑟𝑚 (𝑡)

Where:

● 𝑤 , are weighting coefficients stored in system memory.

1

𝑤

2In certain embodiments, hydrogen sulfide may be assigned a higher weight due to its stronger

association with organic decomposition events. Weighted aggregation ensures that sanitation

classification is not disproportionately influenced by an isolated spike from a single gas sensor.

2.3 Cross-Correlation Analysis

To improve reliability, the system computes temporal cross-correlation between ammonia and

hydrogen sulfide concentration values over a sliding time window.

Sustained sanitation degradation typically produces correlated increases in both gases. In contrast,

transient disturbances—such as brief occupancy or chemical exposure—often result in weak,

inconsistent, or uncorrelated patterns.

If the calculated correlation coefficient exceeds a predefined threshold, the system increases confidence

that a genuine sanitation degradation event is occurring.

This correlation-based validation adds a behavioral dimension to gas measurement rather than relying

solely on magnitude.

2.4 Gas Ratio Pattern Detection

The processor further computes a dynamic gas ratio:

𝑅(𝑡) =

𝑁𝐻

3

(𝑡)

𝐻

2

𝑆(𝑡)

Biological waste events typically produce characteristic ratio bands within expected ranges. By

contrast, exposure to cleaning chemicals or short-term occupancy may cause disproportionate ammonia

increases without corresponding hydrogen sulfide levels.

Ratio-based pattern recognition therefore enables differentiation between sustained biological

degradation and temporary odor disturbances, enhancing classification accuracy.

2.5 Temporal Persistence Filtering

Sanitation degradation is validated only if the composite fusion score exceeds a predefined degradation

threshold for a minimum persistence duration.

If:

𝑆

𝑓𝑢𝑠𝑖𝑜𝑛

(𝑡) > 𝑇

𝑑

for a duration exceeding

, the event is classified as sustained degradation.

𝑇

𝑚𝑖𝑛

Short-duration spikes that do not exceed the persistence window are automatically rejected. This

temporal filtering prevents false alerts triggered by brief or incidental odor fluctuations.

2.6 False Positive Suppression Mechanisms

The system incorporates multiple validation layers to reduce false positives, including:

● Rejection of single-gas deviation without cross-gas confirmation

● Filtering of spikes shorter than a predefined duration threshold

● Correlation validation between gas vectors

● Environmental compensation using humidity and temperature factors

These mechanisms collectively improve signal stability, reduce unnecessary maintenance alerts, and

enhance long-term operational reliability.

2.7 Composite Hygiene Signal Output

The final sanitation assessment signal is computed as:

𝑆𝐻𝑆

𝑟𝑎𝑤

(𝑡) = 𝑤

1

× 𝑁𝐻

3

𝑛𝑜𝑟𝑚 (𝑡) + 𝑤

2

× 𝐻

2

𝑆 𝑛𝑜𝑟𝑚 (𝑡) + 𝑤

3

× 𝐶𝑜𝑟𝑟𝑒𝑙𝑎𝑡𝑖𝑜𝑛(𝑡) + 𝑤

4

× 𝑃𝑒𝑟𝑠𝑖𝑠𝑡𝑒𝑛𝑐𝑒(𝑡)

Where:

● 𝐶𝑜𝑟𝑟𝑒𝑙𝑎𝑡𝑖𝑜𝑛(𝑡)represents the cross-gas correlation coefficient

● 𝑃𝑒𝑟𝑠𝑖𝑠𝑡𝑒𝑛𝑐𝑒(𝑡)represents the time-above-threshold factor

● 𝑤 are weighting coefficients stored in system memory

1

, 𝑤

2

, 𝑤

3

, 𝑤

4

The resulting

is a continuous numerical sanitation state parameter used for hygiene

𝑆𝐻𝑆

𝑟𝑎𝑤

(𝑡)

classification and degradation detection.2.8 Technical Superiority Over Single-Threshold Systems

Conventional single-threshold systems trigger alerts when a single gas exceeds a predefined

concentration level. Such systems are unable to distinguish between sustained degradation events and

short-term odor spikes, leading to excessive false alarms and inefficient cleaning dispatch.

By contrast, the present multi-sensor fusion system integrates:

● Normalization

● Weighted aggregation

● Cross-correlation analysis

● Ratio pattern detection

● Temporal persistence modeling

● Environmental compensation

This multi-dimensional sanitation vector approach produces a stable and reliable sanitation degradation

signal.

As a result, the system achieves:

● Improved detection precision

● Reduced false positive rates

● Optimized maintenance resource allocation

● Enhanced operational efficiency

Through structured sensor fusion and temporal modeling, sanitation assessment is transformed from a

threshold-trigger mechanism into a robust computational classification process.

3.EDGE INTELLIGENCE FOR REAL-TIME SANITATION

PROCESSING

3.1 Functional Edge Processing Architecture

The present invention incorporates an embedded edge-processing module implemented on a

microcontroller-based hardware platform. The edge processor performs real-time sanitation assessment

computations locally, prior to any cloud communication.

The use of edge processing is not a mere architectural distribution of software components. Rather, it is

functionally required to achieve measurable technical improvements, including reduced latency,

optimized bandwidth utilization, and improved operational reliability.

The edge processor is operatively coupled to:

● Gas sensors

● Environmental sensors

● Communication modules

● Local output indicators

The processor executes sanitation classification logic directly on sensor-derived physical

measurements.

3.2 Technical Requirement for Edge Processing

Edge-level computation is technically necessary due to operational constraints inherent in sanitation

monitoring environments:

1. Continuous sensor sampling produces high-frequency raw data streams.

2. Sanitation degradation detection requires immediate computational validation.

3. Public facilities may experience unstable or intermittent network connectivity.

4. Transmission of unfiltered raw sensor data to a remote server would substantially increase

bandwidth consumption and introduce processing latency.

Accordingly, sanitation decision logic is executed locally at the hardware level before data transmission

occurs. This ensures deterministic behavior and consistent performance irrespective of network

conditions.

3.3 Pre-Transmission Processing Operations

Prior to any cloud communication, the edge processor performs a sequence of real-time computational

operations, including:

● Analog-to-digital conversion of sensor signals

● Calibration and normalization of gas concentration values

● Multi-sensor fusion processing

● Cross-correlation analysis between gas vectors● Temporal persistence validation

● Environmental compensation adjustment

● Sanitation Hygiene State (SHS) computation

● Classification into Clean, Moderate Degradation, or Sustained Degradation states

● Event compression and structured data packet generation

Only validated sanitation state transitions or confirmed degradation events are transmitted to the cloud

platform.

This local validation step ensures that raw sensor fluctuations are not directly propagated to remote

systems.

3.4 Data Reduction and Bandwidth Optimization

Gas and environmental sensors may sample at predefined intervals (for example, every 5 seconds).

Continuous transmission of such raw data streams would result in substantial bandwidth usage across

distributed installations.

By performing sanitation classification locally, the system transmits only:

● Sanitized event summaries

● Degradation start and end timestamps

● Computed hygiene state values

● Aggregated trend data, where required

This approach significantly reduces data volume compared to continuous raw streaming.

Edge-level preprocessing therefore achieves measurable bandwidth reduction and network load

optimization, enabling scalable deployment across multiple facilities.

3.5 Latency Reduction and Immediate Response Capability

Effective sanitation monitoring requires near real-time detection of degradation events.

If sanitation decision logic were executed solely in a cloud environment, network transmission delays

could introduce unacceptable response latency.

By computing sanitation classification locally:

● Degradation detection occurs immediately upon threshold validation.

● Alerts can be triggered without waiting for server acknowledgment.

● Local indicators, such as buzzers, warning lights, or display panels, may be activated instantly.

This ensures low-latency response and immediate operational action.

3.6 Reliability During Network Disruption

Public sanitation facilities often operate in environments where network connectivity may be unstable

or temporarily unavailable.

The edge processor maintains autonomous operational capability independent of cloud connectivity. If

communication is disrupted:

● Sanitation classification continues uninterrupted.

● Degradation events are stored in local memory.

● Logged data is transmitted when connectivity is restored.

This autonomous capability enhances system robustness and ensures continuous sanitation monitoring

regardless of external network conditions.

3.7 Alert Flooding Prevention

Temporary odor spikes and environmental fluctuations are common in high-traffic sanitation facilities.

If raw sensor data were transmitted directly to a remote server without local validation, excessive false

alerts could be generated.

Edge-level classification prevents alert flooding by:

● Filtering short-duration spikes

● Rejecting single-gas deviations without cross-gas confirmation

● Validating sustained degradation through temporal persistence modeling

● Transmitting only confirmed sanitation events

This controlled alert mechanism ensures that maintenance notifications are meaningful and actionable.

3.8 Hardware-Integrated Processing Implementation

The edge intelligence module is implemented within a microcontroller comprising:

● Onboard memory storing sanitation models and weighting coefficients

● Sensor interface circuitry● Wireless communication module

● Real-time clock module

The sanitation assessment computations are executed directly by the hardware processor and depend on

physical sensor inputs representing measurable environmental conditions.

The system therefore constitutes a hardware-integrated technical solution rather than an abstract

software algorithm executed independently of physical components.

3.9 Technical Effect Achieved

The edge intelligence module provides demonstrable technical advantages, including:

● Reduced data transmission bandwidth

● Reduced network latency

● Increased system reliability

● Autonomous degradation detection

● Controlled and validated alert generation

● Improved energy efficiency

The edge processing architecture is therefore functionally necessary to achieve measurable technical

performance improvements. It is not merely a relocation of algorithmic logic but an essential

component enabling efficient, real-time sanitation monitoring within hardware-constrained

environments.

4. AI MODEL - OPERATIONAL DISCLOSURE

TO BE CONT…

5.UNCLEAN-PERIOD DETECTION MODULE:

5.1

Detection Mechanism:

The system defines an Unclean Period as a measurable sanitation degradation

interval determined by computed Sanitation Hygiene State (SHS).

5.1.1 Start of Unclean Period:

An Unclean Period is initiated when:

●

The computed SHS value is greater than or equal to 0.6.

●

The SHS remains above this threshold continuously for a minimum duration of 60

seconds.

This persistence-based condition prevents false triggering due to transient odor spikes.

Mathematically:

If SHS(t)>= 0.6 for t >=60 s:

Degradation Start Time = T_degradation

5.1.2 End of Unclean Period:

The Unclean Period terminates when:

●

SHS falls below 0.3

●

The SHS remains below 0.3 continuously for at least 90 seconds.

Mathematically:

If SHS(t) < 0.3 for t >= 90 s

Restoration Time = T_restore

The use of different start and end thresholds introduces hysteresis, thereby improving

detection stability.5.2

Duration Measurement:

The duration of the unclean period is computed as :

T_unclean = T_restore − T_degradation

Where:

T_degradation = timestamp of sustained degradation detection

T_restore = timestamp of sustained hygiene restoration

The system logs this duration in non-volatile memory and transmits it to the cloud database.

5.3

Technical Effect:

The Unclean-Period Detection Module produces a measurable operational parameter,

enabling:

●

Quantifiable sanitation degradation duration

●

Cleaning response efficiency assessment

●

Performance-based maintenance tracking

●

Objective Service-Level Agreement (SLA) compliance monitoring

The generation of a duration-based sanitation degradation metric constitutes a technical effect

beyond mere threshold alerting.

6.

ALERT VALIDATION AND USER FEEDBACK PROCESSING:

6.1

Sensor-Verified Complaint Validation:

When a user submits a sanitation complaint via the interface, the system performs real-time

cross-verification with sensor-derived SHS values.

A complaint is classified as Valid only if:

●

SHS > 0.5, and

●

Persistence of this condition exceeds 30 seconds.

Condition:

If (SHS > 0.5) AND (Persistence > 30 s)

Valid Complaint

Otherwise, the complaint is classified as a Transient Anomaly.

6.2

Technical Contribution:

This mechanism ensures:

●

Elimination of false or malicious complaints

●

Reduction in unnecessary dispatch events

●

Conversion of subjective feedback into sensor-validated technical data

The validation logic operates at system level and is integrated with the edge computation

module.7.

HARDWARE

ARCHITECTURE

(HARDWARE

DEPENDENCE)

The sanitation assessment system comprises:

●

ESP32 microcontroller configured as edge processor

●

Onboard flash memory storing hygiene model parameters

●

MQ-135 gas sensor interface (Ammonia detection)

●

MQ-136 gas sensor interface (Hydrogen Sulfide detection)

●

Wi-Fi communication module

●

Optional GSM communication module

●

5V regulated DC power supply

●

Cloud dashboard server for monitoring and analytics

The system requires physical sensor interfaces and hardware-based computation, thereby

establishing hardware dependence.

8.

DETAILED

EMBODIMENT

(SECTION

10(4)

COMPLIANCE)

8.1

Operational Workflow:

Step 1 - Sensing

Gas sensors acquire NH₃ and H₂S concentrations at a sampling interval of 5 seconds.

Step 2 - Normalization

Raw ppm values are converted into normalized hygiene deviation vectors using

baseline calibration parameters stored in memory.

Step 3 - Sensor Fusion

Edge processor performs:

●

Rolling window averaging

●

Gas ratio computation

●

Weighted SHS calculation

Step 4 - Hygiene Classification

Computed SHS is compared with predefined thresholds to classify hygiene state as:

●

Clean

●

Moderate

●

Degraded

Step 5 - Degradation Detection

Persistence logic determines whether the state qualifies as sustained degradation.

Step 6 - Alert Transmission

Upon validation, a structured cleaning alert is transmitted to the cloud dashboard.

Step 7 - Logging

The calculated Unclean Period duration is stored locally and synchronized with the cloud

database.9.

FLOWCHART DESCRIPTIONS

9.1

Sensor Fusion Flow:

Acquire Sensor Data

Normalize values

Compute gas ratio

Apply weigjted sum

Generate shs output9.2

Hygiene Classification Flow:

SHS Calculation

Compare with Thresholds

Assign Hygiene Class

Store Classification9.3

Alert Escalation Flow

Degraded State Detected

Persistence Verification

Validate Condition

Transmit Alert

Log Event10. BEST MODE DISCLOSURE

The preferred embodiment includes:

Preferred Gas Sensors

●

MQ-135 for Ammonia detection

●

MQ-136 for Hydrogen Sulfide detection

Preferred Sampling Interval

●

5 seconds per acquisition cycle

Preferred Persistence Window

●

60 seconds for degradation confirmation

Preferred Communication Mode

●

Wi-Fi as primary communication medium

●

GSM as fallback redundancy channel